CHEM E 375 COMPUTER SKILLS

LAB 3

GOAL

Setup simple regression examples in Excel and Python. Create functions in Python and learn how to call more complex subroutines.

CHEMICAL ENGINEERING TOPIC

Today we will work with thermodynamic data that relates the vapor pressure of a molecule to its temperature. Vapor pressure is an enormously important property in liquids and relates the equilibrium pressure of a closed gas/liquid system to the interactions between the liquid molecules.

The following data table describes experimental measurements of the vapor pressure of ethane versus temperature:

| T (K) | Pv (Pa) | T (K) | Pv (Pa) |
| --- | --- | --- | --- |
| 80 | 4.98E-02 | 200 | 1.47E+05 |
| 100 | 1.22E+01 | 220 | 2.68E+05 |
| 120 | 3.65E+02 | 240 | 8.91E+05 |
| 140 | 4.02E+03 | 260 | 9.85E+05 |
| 160 | 2.43E+04 | 280 | 1.62E+06 |
| 180 | 6.62E+04 |  |  |

OBJECTIVES

PART 1: EXCEL

- Your boss recommends that you try the Clausius-Clapeyron equation to see if you can get a good fit:

Pv=expA+BT

the equation can also be linearized by taking the natural log

A is positive, B is negative…

- Find the best fit of A and B using the generalized regression method shown in class

A = 		B =

- Make a plot of Pv vs. T and ln P vs 1/T. Compare the experimental data and model. How is the agreement?

- Repeat the regression using the trendline tool in Excel but with linearize data ln⁡(P) vs 1/T

PART 2: PYTHON – using the Lecture 6 notebook

Use the Lab 3 ipynb file as skeleton code and fill in anywhere you see # FILL IN comments. I suggest you refer back to the Lecture 6 Python Demo file as an example of what this should look like, but it’s a completely different function so you’ll need to write those equations yourself. But the general structure/process is the same and much of that process has been given to you. Please make sure you actually understand what each bit of the code is doing so you can do it on your homework, for which you will not be given skeleton code—but you may use Lecture 6 and Lab 3 code as a reference/example.

- In Python create a function that takes A and B as an input and returns the SSR between the experimental data and the model. Use the non-linearized version and the approximate studentized SSR (this means your SSR will be the original version divided by 0.1*Pv)

- Use the minimize function in Python to minimize SSR(A,B)

- Use the curve_fit function in Python (sklearn) to determine A,B. Note that if you are using the non-linearized version of your function, you need to take care about which ‘ydata’ you feed the function. You can effectively use “studentized ydata by calculating Pv,fit/(0.1Pv,fit) and using the ydata Pv,exp/(0.1*Pv,exp). This is a little weird but if you write it out, you will get the point.

- Plot the final curve i

EXTRA LEARNING: Ideas for other things to try in Python

- Repeat w/linearized version

- Try with different error metrics (standard SSR), changing the “10% error approx.”, etc..

SUBMISSION INSTRUCTIONS

Please PDF your notebook and upload BOTH that PDF (.pdf) and the Jupyter Notebook itself (.ipynb), as well as your Excel sheet (.xlsx/.xls) to Canvas under the Lab 3 assignment for a completion grade. You can PDF your Jupyter notebook in several ways (ex: “Export and save as”).

Please remember to submit before the deadline!