Experimental Request

From: Alexis Prybutok, CTO

To: Engineering Team

Subject: Heat Exchanger

Our company is investigating a waste heat recovery device for residential bathroom showers. To keep costs down we think a concentric tube heat exchanger is the best option. The fabrication team got ahead of themselves and made a demo unit without any planning or analysis. We would like you to characterize their demo unit so we can use the data to build a more refined second iteration. The fabrication team shared the following info:

| Inner Tube | Inside diameter | 0.0083 m |
| --- | --- | --- |
|  | Outside diameter | 0.0095 m |
|  | Material | stainless steel |
| Outer Tube | Material | clear plastic |
| Heat Transmission Length | (per tube) | 0.66 m |

You will need to characterize the range of overall heat transfer coefficients, U (W/m2K), over a wide range of flowrates and we need to optimize the length to keep costs down and to make it fit in existing drain space.

Using this demo unit, we would like you to do the following:

- Determine if co- or counter-flow works better here. Pick an intermediate flow rate for the hot and cold streams, and determine the temperature profile for each stream down a 4-tube-length co-current vs counter-current configuration. Compute the overall heat transfer coefficient for each configuration.

- Select either co-current or counter-current flow in your 4-tube-length system, and
determine the overall heat transfer rates for a full-factorial combination of 3 different flow rates for both the hot and cold flow.

- Using the same configuration as one of your experiments in (2) determine the impact of shortening the heat transfer to 2-tube-lengths on the overall heat transfer coefficient.

- Anything else you think we should test.

- Estimate the heating cost savings of a typical shower. If we wanted this device to pay itself back in a year, how much can we charge for it?

Please prepare your findings in a formal report for our internal library.