Experimental Request

From: Alexis Prybutok, CTO

To: Engineering Team

Subject: Pipe Friction Factors

Industrial electricity rates are rising rapidly in our region, so we are keen to understand factors that impact our pumping energy costs before adding capacity to our plant.

In particular, a new piping and valve supplier claims they offer the lowest frictional loss piping in the business, and even sell a high-tech lined tube they claim is “biomimetic shark skin” with the lowest friction factors ever measured. They also offer advanced valves they claim to be virtually friction loss-free. I don’t think I believe it, but their sales team talked to the CEO all morning and now she is convinced we should use it for our upcoming expansion.

To help verify or refute the supplier claims, we have built a testing apparatus that uses water to directly measure pressure drop in the vendor’s different straight pipe products, different kinds of pipe bends they offer, and across their so-called advanced valve products. The various pipes in the apparatus have the following specs:

| Pipe Outer Diameter (OD) (mm) | Pipe Inner Diameter (ID) (mm) |
| --- | --- |
| 6.4 | 4.5 |
| 9.5 | 7.7 |
| 12.7 | 10.9 |
| 19.1 | 17.2 |
| 19.1 | 15.2 (shark) |

Using this apparatus, we would like you to report the following:

- A calibration curve for the orifice meter to determine flow rate in the test device. Test the venturi meter too. How well do the meters compare to theory? Is one better?

- Fanning Friction Factors vs a wide range of Reynolds number for all the vendor’s piping products. Plot your results against standard curves, such as Fig. 6-9, page 6-10 of Perry’s Chemical Engineers Handbook, 8th Ed. (see figure below) to evaluate smoothness.

- Assess the frictional losses for the various pipe bends and valves offered by this vendor and compare them to standard values reported in the literature.

This report will go into our internal library, so please make it look good.