# GAS-LIQUID ABSORBER UNIT

# STANDARD OPERATING PROCEDURE

TABLE OF CONTENTS

UNIT INFORMATION	2

PREPARING SOLVENTS AND THE COLUMN FOR THE LAB	3

STUDYING THE HYDRODYNAMICS OF THE COLUMN (FINDING THE COLUMN LIMIT)	4

STUDYING COLUMN EFFICIENCY BY ABSORBING CO2	4

SHUTTING DOWN AND CLEANING OUT THE COLUMN	6

# UNIT INFORMATION

Gas-liquid absorbers are important industrially for waste from exhaust streams, or other important separations. In order to use them effectively, we must characterize column operation and efficiency, and we must optimize for quantity and quality.

The goal of this unit is to clean CO2 out of exhaust. The column as two sections—you feed in gas (air and/or CO2) on the bottom of the column and inlet solvent from the top of the column (just water or NaOH solution). Note that often in industry there is a regeneration column to regenerate the solvent, but this pilot column does not have a regeneration unit. The column has a 2-inch diameter and is filled with 0.2 inch glass raschig rings (6 mm diameter + length).

We will use a reactive absorption to absorb CO2 that is as follows:

2 NaOH aq+CO2→Na2CO3aq+H2O

Note however that the reactions actually occur in steps. The NaOH will separate into ions Na+ and OH-, and combined with the ions from water (H+ and OH-), will then react as follow:

Gaseous CO2 will dissolve in water as:

CO32-+H+→HCO3-

OH-+H+→H2O

The H+ ion reacts with the OH- ions in excess and with the CO32- ions.

The moles of bicarbonates formed are equal to the moles of carbonate ions present.

nHCO3- formed=n(CO32- present)

nH+=nOH-+nCO32-

And then the bicarbonate (also known as carbonic acid, HCO3-) ion reacts with the H+ ions:

H++HCO3-→H2CO3

This product can then separate into ions and interact with NaOH to form Na2CO3 and water. Thus, the product in the waste container will be primarily carbonic acid (H2CO3), Na2CO3, plus some NaOH solution from whatever does not react and the Na+ ions interacting with water. Additionally, the column will be warm since it’s a reactive absorption.

As the gas travels up the column and interreacts with the solvent via contact with the raschig rings, the solvent absorbs the CO2, the percent composition of which in the gas is measured via an infrared CO2 analyzer attached to a vent. Note two things: to prevent damage to the analyzer, 1) the vent has a Millipore filter reduce water contact with the analyzer (humidity), and 2) one should not flood the column.

The long tube at the bottom of the column is used to keep the level constant automatically by being a specifically designed length to accumulate liquid. Note that the further down the level in this bottom tube goes, the more back pressure there is in the column.

There are two manometers on the column to measure the pressure of the column. The right manometer (ΔP inlet) reads the pressure difference across the column from the top to the bottom of the column itself. The left manometer (ΔP inlet) measures the pressure difference between the atmosphere and the bottom of the column. To get the total pressure of the column, one must add the two pressures together (ΔP column + ΔP inlet).

The pump used to flow solvent from the tank into the column is a volumetric pump and is very unsteady. To avoid unsteady flow rates, there is a flowmeter and vessel on the back (with outlet valve V1) of the column that holds both gas and liquid to adjust the flow rate as the pump changes since the air compresses.

Below are the instructions for:

- Preparing solvents and the column for the lab (must be done each week).

- Studying the hydrodynamics of the column (Week 2 objective).

- Studying the column efficiency of absorbing CO2 with different solutes and operating conditions (Week 2 objective with water & Week 3 objective with NaOH).

- Shutting down the column (must be done each week).

# PREPARING SOLVENTS AND THE COLUMN FOR THE LAB

- Analyze the inlets, outlets and all tubing of the column to determine what each portion is for.

- Map the inlets, outlets, valve, and other components to the P&ID diagram (note the P&ID contains a riser that is not on device).

- Prepare 1 mol/L solution of sodium hydroxide (NaOH) in 10 L of deionized (DI) water. (Note, only needed if studying the solvent efficiency with NaOH, not needed for hydrodynamics of the lab or efficiency with just DI water).

  - Fill 10 L DI water into a Nalgene tank.

  - Add 584 g NaOH.

- Prepare the manometers.

  - Close valves (V8 and V9) on the bottom of the manometer.

  - Fill the manometers with DI water.

    - Note: the glass T-junction stopper used to connect the two manometers is extremely fragile! Please ask for assistance if necessary.

  - Add the small glass device to connect the top of the two manometers but ensure there is no force applied and it is not too tight as not to break the glass.

- Verify sample valves and the drain valve are closed (V4, V5, V6 and V7)

# STUDYING THE HYDRODYNAMICS OF THE COLUMN (FINDING THE COLUMN LIMIT)

The first thing we need to do is study the limit of the column, or in other words determine the hydrodynamics of the column, with only water and air (i.e. no CO2 or NaOH). We need to find the limit of the column based on the flow rates of gas (air) and water, where the limit will be indicated by two changes: 1) a visual indicator and 2) a parameter you can track.

- Verify the skid is plugged into the wall.

- Ensure the valve to CO2 analyzer is closed (V13).

- Put the pump’s inlet pipe (line with filter) and purge line (line without filter) into the DI water tank.

- Put the outlet line from valve V12 into a labeled waste container.

- Turn on the pump by pushing the power button.

- Vent the pump of air by turning the valve at the top of the pump so air outlets into inlet fluid tank. This is known as “priming” the pump. When all air has been purged, return the valve back to the starting position to send fluid into the column.

- Turn the dial (stroke) to 40 and set the duty cycle (buttons) to 100%.

  - Note: Only adjust stroke (dial) when pump is on or you will damage the membrane.

  - For general operation, we recommend setting the stroke to 30 and operate at a high duty cycle. Generally, a lower stroke and high duty cycle is preferred to keep the pump in good working condition for longer.

- As the pump is working, the surge vessel in the back will fill. While this is happening, loosen the vessel lid/screw. Allow the vessel to fill to 50% liquid, then tighten the lid slightly. This vessel helps stabilize the flow rate of liquid into the column.

- Set the liquid flowmeter (F1) to 15 L/hr by adjusting the pump stroke.

- Open the valve that outlets the graduated receiver into the solvent waste tank (V12) to drain the receiver.

- Ensure the house (lab) air is line is connected to the air inlet line (black tube from ceiling connected to blue line).

- Ensure the air valve in the back of the column is open (it should be already, but ask a TA if necessary to verify).

- Adjust flow rate of air using the flowmeter (F2) to 10 NL/min.

- Slowly increase the air flow rate in the column until the column begins to flood, at which point you have found the limit of the column. Take note of the gas and water flow rates at the flood limit. (Generally, this starts to happen at 50 NL/min gas and higher, and at 15 L/hr water). Determine the visual and parameter indicators of flooding.

- Do not let the column flood above the halfway point. To stop flooding, decrease air flow rate.

- Record liquid and vapor flow rates to determine the optimum regime such that you can operate the column without flooding. Note the maximum vapor flow value (flooding limit) may change with different liquid flowrates.

# STUDYING COLUMN EFFICIENCY BY ABSORBING CO2

## If starting the column for the first time today

- Verify the skid is plugged into the wall.

- Ensure the valve to CO2 analyzer is closed (V13).

- Put the pump’s inlet pipe (line with filter) and purge line (line without filter) into the DI water tank.

- Put the outlet line from valve V12 into a labeled waste container.

- Turn on the pump by pushing the power button.

- Vent the pump of air by turning the valve at the top of the pump so air outlets into inlet fluid tank. This is known as “priming” the pump. When all air has been purged, return the valve back to the starting position to send fluid into the column.

- Turn the dial (stroke) to 40 and set the duty cycle (buttons) to 100%.

  - Note: Only adjust stroke (dial) when pump is on or you will damage the membrane.

  - For general operation, we recommend setting the stroke to 30 and operate at a high duty cycle. Generally, a lower stroke and high duty cycle is preferred to keep the pump in good working condition for longer.

- As the pump is working, the surge vessel in the back will fill. While this is happening, loosen the vessel lid/screw. Allow the vessel to fill to 50% liquid, then tighten the lid slightly. This vessel helps stabilize the flow rate of liquid into the column.

- Set the liquid flowmeter (F1) to 15 L/hr by adjusting the pump stroke.

- Open the valve that outlets the graduated receiver into the solvent waste tank (V12) to drain the receiver.

- Ensure the house (lab) air is line is connected to the air inlet line (black tube from ceiling connected to blue line).

- Ensure the air valve in the back of the column is open (it should be already, but ask a TA if necessary to verify).

- Adjust flow rate of air using the flowmeter (F2) to 10 NL/min.

## If continuing from hydrodynamics study steps

- Turn off pump via power button.

- If switching to NaOH as solvent you will need to drain the system of water:

  - Unscrew inlet and purge pump lines to drain them of water.

  - Unscrew and drain level vessel in back by opening valve V1.

  - Collect DI water and dump it down the drain.

  - Drain the column by opening valve V5 at bottom. Since this is water, you can pour this waste down the drain.

- Switch the pump’s inlet line (line with filter) and purge line (line without filter) to the desired solvent tank if necessary (either 1M NaOH or DI water).

- If switching solvents, put the outlet line from valve V12 into the correct waste container.

- Turn on pump via power button.

- Prime the pump if necessary using the screw valve at the top of the pump to purge air into the inlet fluid tank. When all air has been purged, return the valve back to the starting position to send fluid into the column. Note: likely not necessary to prime if you already used equipment today.

- Adjust pump stroke to 30 and duty cycle high.

- Set the liquid flowmeter (F1) to 15 L/hr by adjusting the pump stroke.

## Continuing the efficiency study (from either of the above starting points)

Normally you will run this experiment at 12-15% CO2 (the rest of the gas being air), and the analyzer will get down to somewhere around 2-5%, meaning it absorbs ~80% of CO2. In order to achieve this volume percent of CO2, you can either slowly work up to that percentage, or operate at it from the start, either way keeping in mind the maximum total gas flow rate you should not exceed based on your hydrodynamic study of the column.

- The CO2 analyzer should be at 0 to start, and it takes ~1 minute to reset. Note that when you finish with the lab for the day, you need to inject air to get the analyzer to read 0 again, or CO2 will be stuck in the tube and the CO2 analyzer will be recalibrated at 0 incorrectly at the start of the next period.

- Open valve for CO2 analyzer (V8).

- Turn the air flow rate 40 NL/min. Note that we will use no higher than a 50 NL/min maximum total gas flow rate per the hydrodynamics study.

- Turn the CO2 tank regulator to between 15-18 psi. NOTE: Let the TAs do this step.

- Turn CO2 flow rate to 1 NL/min. This will mean we have a 2% CO2 composition at the inlet calculated by the following equation:

% CO2=vCO2vair+vCO2 x 100%

this is showing 0.5 % (% volume, very precise) on analyzer after absorbed

- Allow the analyzer to stabilize before taking a measurement of the CO2 absorbed. At the above conditions, the analyzer showed 0.5% (note this is a record of % CO2 by volume, and it is very precise).

- Increase the CO2 flow rate to 1.5 NL/min. Wait for value of analyzer to steady and record the value.

- Increase the CO2 flow rate to 2 NL/min. Wait for value of analyzer to steady and record the value. Note that at this point the temperature is increasing because the absorption is reactive and the reaction is occurring.

- Increase the CO2 flow rate to 3 NL/min. Wait for value of analyzer to steady and record the value.

- Increase the CO2 flow rate to 4 NL/min. Wait for value of analyzer to steady and record the value.

- Note that now we are at the limit of the CO2 flowmeter (and operating at 44 NL/min gas total), so in order to achieve higher % CO2 at the inlet, we must decrease the air flow rate. Lower air flow rate to 35 NL/min, keeping the CO2 at 4 NL/min. Wait for value of analyzer to steady and record the value.

- Lower air flow rate to 30 NL/min, keeping the CO2 at 4 NL/min. Wait for value of analyzer to steady and record the value.

- Lower air flow rate to 20 NL/min, keeping the CO2 at 4 NL/min. Wait for value of analyzer to steady and record the value.

- At this point, you can do a mass balance to figure out the column efficiency, knowing the inlet and outlet composition.

- Note that you could take samples from middle of the column to get understanding of separation across the column.

# SHUTTING DOWN AND CLEANING OUT THE COLUMN

## If you ran the column with only DI water and no NaOH

- Turn off the pump via the power button.

- If you used CO2, turn off the CO2 flow rate using the flowmeter (F2), turn the air flow rate to 40 NL/min using the flowmeter (F1) and wait until the analyzer reads 0%.

- Unscrew inlet and relief pump lines to purge them of water.

- Unscrew and drain level vessel in back by opening valve V1. Since this is DI water (from hydrodynamics study), you can empty this into a beaker and dump it down the drain.

- Drain the column by opening valve at bottom. Since this is water, you can pour this waste down the drain.

- Drain the manometer water by opening valves at the bottom of each (V8 and V9). Dump DI water down the drain.

- Close the valve to the CO2 analyzer (V13).

- Open the sample vents and the bottom valve (V5, V6, and V7) and leave them open so the column can dry.

- If used CO2, turn off the gas cylinder and close the black valve at the back of the column to purge air NOTE: Have the TAs do this step!

- Close the air valve on the regulator at the back of the skid (ask TA for help if necessary).

## If you ran an efficiency study with NaOH and CO2 gas

The column now needs to be cleaned of CO2 and it is imperative to clean out the CO2 analyzer to ensure it zeros correctly at the start of the next period.

- Turn off the CO2 flow rate using the flowmeter (F2).

- Turn the air flow rate to 40 NL/min using the flowmeter (F1) and wait until the analyzer reads 0%.

- Turn off the air flow by turning the flow rate to 0 NL/min.

- Turn off the pump via the power button.

- Unscrew inlet and relief pump lines and clean these in the sink. Note NaOH rinsate is caustic, be sure to wear gloves!

- Unscrew and drain level vessel in back by opening valve V1. If this is pure 1M NaOH solution you can pour it back into NaOH vessel for reuse.

- Put the pump’s inlet line (line with filter) and purge line (line without filter) into the DI water tank.

- Put the outlet line from drain valve (V12) into a labeled waste container.

- Turn on the pump by pushing the power button.

- Pump DI water for 10 minutes to purge all non-water out of the system.

- Drain the column by opening valve V5 at the bottom. You can pour this into the waste container.

- Close the valve to the CO2 analyzer (V13).

- Open the sample vents and the bottom valve (V4, V5, V6, and V7) and leave them open so the column can dry.

- Open the sample vents and the bottom valve (V4, V5, V6, and V7) and leave them open so the column can dry.

- Drain the manometer water by opening valves at the bottom of each (V8 and V9). Dump DI water down the drain.

- Turn off the gas cylinder and close the black valve at the back of the column to purge air. NOTE: Have the TAs do this step!

- Close the air valve on the regulator at the back of the skid (ask TA for help if necessary).