# REVERSE OSMOSIS MEMBRANE UNIT

# STANDARD OPERATING PROCEDURE

TABLE OF CONTENTS

UNIT INFORMATION	2

PREPARING SOLVENTS AND THE COLUMN FOR THE LAB	2

DETERMINING THE OSMOTIC PRESSURE OF TAP AND SALT WATER	3

DETERMINING EFFECT OF VARYING SALT CONCENTRATIONS	3

SHUTTING DOWN AND CLEANING THE UNIT	3

# UNIT INFORMATION

Membranes can be used to purify contaminants from water, such as for applications of purifying salt water to create fresh water. Characterizing the efficiency of your membrane is critical for optimizing the process to generate sufficient quantity and quality of purified product.

The unit is equipped with a membrane (white tube) that is capable of purifying sea water (>10 g/L NaCl water) to a quality better than deionized (DI) water used in laboratory settings. The membrane should last several years if cared for properly. The unit also has two filters (5 M and 10 M) that pre-filter city water before it enters the membrane tank in order to further protect the membrane.

The pump on the unit is a positive displacement pump that is powerful enough to generate the power necessary to pump fluid through the membrane. Pressure is not controlled directly but will remain constant depending on the configuration in use. The relief valve is set to 40 bar. Note: never run the pump dry or you will damage the pump.

The feed tank is equipped with a coiling coil because the pump generates heat due to its high pressure (temperature can go up 10-20C over the course of an hour). The cooling water comes from the city and empties into the drain.

Water purity will be determined by measuring conductivity. After filtering through the membrane, the permeate (clean water), which should have a low conductivity and flows toward flowmeter FI2 (maximum of 50 L/hr), will flow separately from the retentate (dirty water) that flows toward flowmeter FI1 (up to 600 L/hr). There is an inline conductivity probe for the permeate, and a separate hand-held conductivity probe for the retentate. Either or both the permeate and retentate can be collected in waste/discharge tanks or reconnected back to the feed tank for recirculation.

Below are the operating instructions for:

- Preparing solvents and the column for the lab (must be done each week).

- Determining the osmotic pressure of tap and saltwater to calculate the membrane polarization (Week 2 objective).

- Determining the effect of varying pressure and saltwater concentrations on osmotic pressure, membrane polarization, and other membrane characteristics (Week 3 objective).

- Shutting down and cleaning the unit (must be done each week).

Note, all background, theory, equations, and analytical process steps will need to be determined by the team.

# PREPARING SOLVENTS AND THE COLUMN FOR THE LAB

- Fill the feed tank with either tap water (V1 and V4 open, V3 closed) or a salt water solution (low is <200 mg/L, high is >1 g/L but no greater than 10 g/L solution of NaCl in tap water).

- Make sure valves V2, V5, V7, and V10 are closed.

- Make sure valve V9 and V17 is open.

# DETERMINING THE OSMOTIC PRESSURE OF TAP AND SALT WATER

- Turn on the machine using the main switch on the control box.

- Ensure valve V6 is open.

- Open valve V9 to ensure outflow of tank will flow back into the tank to start.

- Maximize flow through flowmeter FI1 by opening V17.

- Close valve V10 to ensure nothing will flow into the membrane.

- Close valves V7, V2, V5 to ensure tank does not empty.

- Turn on the pump P1.

- Open valve V10 to enable flow to enter membrane.

- Start closing/adjusting V9 and FI1 until you meet the following criteria:

  - Conductivity of permeate is 0-1 S.

  - Both flowmeters, FI1 and FI2, have flow rates only within their respective bounds.

  - Pump PI2 is less than 40 bar.

    - If the pump runs more than 40 bar, it will send fluid back to the tank.

- Turn on cooling water by opening water valves V1 and V3 (V4 closed), ensuring outlet empties into the drain.

- During handling, the operator can easily stop the pump by pressing the red "STOP PUMP"

button after bringing back the membrane at zero pressure:

- Slowly but fully open V9

- The pressure in the module decreases.

- Open V17 if necessary

- Close V10

- Stop P1

WARNING: In case of emergency, the pump can be stopped by pushing the red button “STOP PUMP”. Otherwise, always decrease the pressure in the module before stopping the pump.

# DETERMINING EFFECT OF VARYING SALT CONCENTRATIONS

- Follow the above steps with varying concentrations of salt water, no higher than 10 g/L.

# SHUTTING DOWN AND CLEANING THE UNIT

WARNING: Salted water is very corrosive. You MUST rinse the installation immediately after

using salted water, whatever the concentration.

Do all of the following steps in the following order.

## Rinsing the feed tank directly.

- Drain the tank of any water in it.

- Fill the feeding tank with 20-30 L of water.

  - Open V2 to drain tank. Close V2 when drained.

  - Replace tank with clean water (not salt water) by opening V1 and V4.

  - Close V1 and V4.

  - Repeat if needed.

## Rinsing the pump directly.

A circuit allows the suction of the pump to be fed directly from the city water, without going through the tank.

- Close all valves.

- Drain the tank of any water in it by opening V2. Close when done draining.

- Check that V5, V6, V10 and V12 are closed.

- Open V2, V7, V9 and V11.

- Open V1.

- Wait until your tank has over ~15 L in it.

- Start P1, fed with the tap water in direct passage, the pump circuit is rinsed. If the flowrate and pressure in the water network are sufficient, water flows to the module and out in the feeding tank

- Run for several minutes and check conductivity in the tank.

- Stop the unit.

- Empty the tank.

- Close all valves.

NOTE: Check that the tank does not overflow. As needed, open V2 to drain it during this process.

WARNING:  In order to preserve the pump, the water flowrate must be 1000 L/h at least. This operation is necessarily supplemented by a batch rinse on the feeding tank to clean all the circuits of the unit.

## Rinsing the membrane module directly.

We will rinse the membrane using water directly from the city and empty the contents into the tank so we can measure the conductivity.

- Close all valves. Note the pump does not turn on during this process. Do not turn the pump on at any point.

- Make sure the tank is empty. If needed open V2 to empty, then close.

- Remove the “Backflush Only” warning tag from V14 by removing the carabiner.

- Put V14 in position to connect FI3 outlet to the module.

- Make sure V4, V7, V12, and V13 are closed.

- Open V9 and V10.

- Open V1 and V11.

- To avoid water hammer, slowly open V12 (F13)

- Set a flowrate of F13 between 200 and 400 L/h.

- Check conductivity in the module outlet (draining point, the tank).

- When the conductivity in the tank is equal to the tap water conductivity, close V12 (F13).

- Close V1.

- Empty the tank by opening V2.

- Close all valves.

- Return V14 to its original position and re-add the warning tag to prevent it from rotating.

WARNING: The flow is reversed compared to the production mode of the module. Do not cause

water hammer or rise of pressure in the module. This operation is necessarily supplemented by a batch rinse on the feeding tank to clean all the circuits of the unit.

## Rinsing the module in batch mode.

- Close all valves.

- Open V6.

- Fill the feeding tank with 20-30 L of water.

- Open V2 to drain tank. Close V2 once drained.

- Replace tank with clean water (not salt water) by opening V1 and V4.

- Keep V4 open, and use V1 as needed to keep the volume of the inlet tank >15 L.

- Put the permeate and retentate hose in their respective tanks (not the feed tank) to work in batch mode.

- Open V9 and V17 (FI1).

- Close valve V10 to ensure nothing will flow into the membrane.

- Close valves V7, V2, V5 to ensure tank does not empty.

- Turn on the pump P1.

- Open valve V10 to enable flow to enter membrane.

- Put the module under pressure with ΔP of 20 bar approximatively.

- Let it run for approximatively 10 minutes.

- Stop the unit by doing the following:

  - Slowly open V9 fully and V17 fully.

  - Close V10.

  - Turn off P1.

- Measure the conductivity in the feeding tank.

- Drain the tank.

Repeat this operation several times until the water remains clear and the conductivity in the

feeding tank is equal to that of the tap water with which the tank is filled.

## Draining all tanks in the system.

- To drain the feeding tank, open V2.

- To drain the retentate tank, open V20.

- To drain the permeate tank, since the tank is small, you can drain it by reserving it or by using a hose to siphon it.

- To completely drain all circuits, use the following steps:

- Remove the cap from V8 and open it, let it flow then close V8 and put the cap back.

- Remove the cap from V16 and open it, let it flow then close V16 and put the cap back.

- Open V19 and let it flow.

- To drain the rinsing circuit, open V13, let the water flow and close V13.