# REACTOR UNIT

# STANDARD OPERATING PROCEDURE

TABLE OF CONTENTS

UNIT INFORMATION	2

PREPARING SOLVENTS AND THE COLUMN FOR LAB	3

STUDYING RESIDENCE TIME DISTRIBUTION OF A REACTOR	4

STUDYING EFFECT OF AGITATION SPEED AND FLOW ON DTS	5

SHUTTING DOWN AND CLEANING OUT THE UNIT	5

TROUBLESHOOTING	6

# UNIT INFORMATION

Reactors are important industrially for chemical production, but choosing the right reactor for the job involves understanding theory for various types of reactors, and characterizing reactor flows and composition profiles. When you conduct a reaction, you need to know how long the molecules are in contact, where if the contact time is too low (flow is too fast), the reaction will not be complete, but if it is too slow you are losing time and product by not operating efficiently.

In any given reactor, the molecules stay for a period of time (ts, residence time), which depends on the hydrodynamic profile and the geometry of the reactor, which can be different from the average residence time, τ. Thus, the residence time distribution (DTS) influences the chemical performance of the reactor. To experimentally determine the DTS, the method using tracers consists in marking the molecules entering the reactor and following what happens to them until they leave the reactor (enumerating them in the flow at the output as a function of time).

The unit consists of four reactors of equal volumes:

- One large, stirred 0.8 L CSTR with a heating jacket (R2, but on far left, first in line).

- Two 0.4 L CSTRs in series (adding up to a total of 0.8 L), otherwise known as cascade reactors (R1, second in line).

- One small-diameter 0.8 L PFR with strong axial dispersion (R3, third in line). Note this reactor only operates at plug, rather than laminar, at certain flow rates.

- One large-diameter (2 in) 0.8 L PFR with a heating jacket. This reactor is packed with solid glass beads such that the space between beads adds up to 0.8 L.

The unit is also equipped with temperature and conductivity probes. Since the conductivity of deionized water is 0, the conductivity probes can be used to track the tracer, for which we will use saturated salt water. This will enable us to determine the residence time distribution across the reactors and compare their profiles. Changing the agitation speed or flow rates will change the residence time distribution (DTS).

Data acquisition is automatic via the probes on the unit and the control box. Data is collected every 2 seconds. The time on the computer is real time. Conductivity probes have large range, up to 50 or 60 milli-siemens (mS), so they can go up to very high conductivity value. Lower concentrations will exhibit lower accuracy. Note that you will be below detection limit at 5-6 S. The data acquisition will automatically switch from S to mS for the reading on the front of the control panel, and the graphs shown on the panel are in units of mS.

Other operating conditions and parameters of use include the following:

| PARAMETER | VALUE | UNIT |
| --- | --- | --- |
| Feed operating flow rate range | 2.5-25 | L/hr |
| CSTR stirring speed range | 0-1000 | rpm |
| CSTR impeller dimensions | 12.7 x 30.5 mm (0.4 x 1.1 in) | mm (in) |
| R3 nominal diameter (DN) | 15 | mm |
| R4 nominal diameter (DN) | 50 | mm |
| Feed and waste reservoir volume | 30 | L |

The unit is also equipped with a control box containing a touch screen, which is connected to a computer to enlarge and control the screen, and automatic data acquisition that can be obtained via a USB drive.

- Preparing solvents and the column for the lab (must be done each week).

- Determining the residence time distribution (DTS) of each reactor (Week 2 objective).

- Determining the effect of agitation speed and flow rate on DTS for each reactor (Week 3 objective).

- Shutting down the column (must be done each week).

# PREPARING SOLVENTS AND THE COLUMN FOR LAB

- Analyze the inlets, outlets and all tubing of the column to determine what each portion is for.

- Map the inlets, outlets, valve, and other components to the P&ID diagram.

- Note: you can skip this step as this has been done for you with the attached lab computer. Connect to RAP screen with RealVNC software and ethernet-USBC adapter and by changing IP address of the computer to match.

- Make a 50 mL saturated solution of sodium chloride (NaCl) in deionized (DI) water.

- Prepare a full tank of DI water and put all pump inlet tubes inside the container.

- Prepare an empty waste container and put all reactor outlet tubes inside the container.

- If using R3 or R4, insert injection port into the respective screw hole location. The injection ports are located in the Styrofoam block held inside the DAQ box.

- Open only the valves leading to the reactor(s) you intend to use as follows:

  - If using pump 1 (P1, left), open valve V3 for R2, V4 for R1, V5 for R3, V6 for R4.

  - If using pump 2 (P2, left), open valve V13 for R2, V14 for R1, V15 for R3, V16 for R4.

- Insert the temperature probe (metal probe on white cord) and conductivity probe (black ports, need to plug in probes) into the reactor you will use using the instructions below (ports labeled on P&ID as T1I and ΩI, respectively). Conductivity probes can be found in test-tube holders near unit. Conductivity probes are stored in a KCl solution.

  - Conductivity probe

    - Unscrew black top on the conductivity probe and store top somewhere safe.

    - Screw the probe into the black port on the reactor but take care not to tighten too much.

    - Put the conductivity probe into the reactor via port in the lid/on top of the reactor such that it is submerged in fluid. You need to be careful when inserting the probe such that you do not break it, as these are delicate and expensive. You can adjust or rotate white lid to do so.

  - Temperature probe

    - Insert the temperature sensor (left small hole) in the reactor lid/top.

- Note for cascade reactors (R1), you can have two probes, one in each reactor and you can view data for both at once on the screen.

  - Red is 1st probe (far left on unit).

  - Green is 2nd probe (far right on unit).

# STUDYING RESIDENCE TIME DISTRIBUTION OF A REACTOR

Follow this procedure for one reactor at a time, eventually going through all four reactors. Note that we need to fix the flow rates to be continuous for any given reactor. This is achieved using the vessels at the top to regulate the flow rate such that even if the level in the tank changes, the flow rate will remain the same.

- Turn on the main switch on the control panel. Conductivity and temperature probes will relay data to the GUI.

- During operation, feed will flow from a tank on bottom, through the pump, up to level vessel up top of the unit, down to rotameter on front of the unit, where you can set the flow rate (FI1 or FI2), then into the reactor.

- Ensure the inlet flow tank only contains DI water.

- Ensure the outlet flow tank is a separate, empty waste tank.

- Ensure the overflow outlet tube from the level vessel is inside the empty waste tank.

- Ensure the valves are open only to the reactor you are using.

- If using R1 or R2, ensure or adjust the pipe on bottom of the reactor and sticking up into the tank is at same level as volume line (0.8 L for R2 or 0.4 L for each reactor in R1) to ensure the volume is correct.

- Turn on pump by turning off standby (make sure you are using the correct pump for the valve positions you’re using and reactor of choice).

- Set flow controller (FI1 or FI2) to 10 L/hr.

- Fill the reactor with DI water by letting the pump run.

  - Note for PFR R3:

    - Make sure all drains (V18, V20, and V22) and vents (V19 and V21) are closed before starting.

    - Turn on corresponding pump to fill with DI water.

    - Vent by slightly opening the valves at the top (V19 and V21) while filling PFR. Close the vents once filled.

    - Note that for this reactor, probes are located at the outlet of the reactor, so data collection may take a while. Probes are at end so will take a while

- Note that in general, if the pump is not working, disconnect pump line from level vessel and then reconnect.

- If using R1 or R2, turn on agitator with RPM tuner on CSTR.

  - Use 200 RPM minimum. Note that 300 RPM is close to perfectly stirred, but if you set the value to be too high you will create a vortex and lose efficiency.

- Select reactor in use on the on control panel screen.

- Prepare a 5 mL sample of saturated NaCl in DI water using a syringe to use as a tracer.

- Simultaneously inject the 5 mL tracer, hit Start on the control panel at same time (light will turn blue when Start is hit), and record the time on your computer (including seconds). Note, this will require at least two people, one for each part, and precise coordination. Hitting start will show a vertical line on the graph where the time point of data collection began.

  - Note for R3: injection will require opening the valve on the port (V17), injecting the sample, and then closing the valve on the port.

- Collect data over time. This may take a while depending on how long the tail of the injection is.

- Retrieve data by plugging USB into port on control panel.

## Using the heating jacket

- Fill heater with DI water or coolant.

- Turn start key on emergency switch on control box.

- Hit start.

- Turn on heater (left side of unit).

- Use S button to change setpoint of the following items:

  - Temperature

  - Fan (doesn’t change)

  - Timer

  - (something)

  - Ramp

- Use F to change function of things like select medium (water).

- You should expect ΔT between Tjacket and Treactor of 5-10C.

# STUDYING EFFECT OF AGITATION SPEED AND FLOW ON DTS

- Repeat steps 9-26 with a different agitation speed (RPM) and/or flow rate (FI1 or FI2).

# SHUTTING DOWN AND CLEANING OUT THE UNIT

- Flush DI water through any and all injection ports used to ensure salt is cleared from the device so it does not corrode the ports.

- Wipe down the agitators (portion above the CSTR lids) with a wet KimTech wipe to prevent salt buildup.

- Flush DI water through the entire system for 5 minutes to get all salt out.

- To drain reactor, unscrew bottom valve (V10 for R2, V8 and V9 for R1, valves on bottom of R3, V25 for R4) and can drain to 0 volume.

  - Use an empty beaker to collect fluid. Place beaker near the drain valve.

  - Open the valve to drain.

  - Close once empty.

  - Dump waste fluid down the drain.

- Drain level containers.

- Drain all pumps.

- Disconnect conductivity probes and store in KCl solution.

- Dispose of waste tanks down drain.

- Be sure to export your data onto the designated flash drive before you leave lab.

  - Insert the drive into the port on the control box.

  - If something pops up prompting you to download or upload anything, click X to stop it.

  - Click on the plot button on the screen.

  - Click Today.

  - Make sure “Exporting in progress” pops up. Once that goes away, you can unplug the USB and it will be inside the folder on the drive and labeled with the data.

# TROUBLESHOOTING

## Flowmeter is unresponsive to changes in the dial

In this case, we need to “burp” the pump.

- Close all valves directed to any reactor from the flowmeter (V2-6 or V12-16).

- Open the valve to a reactor not in use.

  - If R4 has not arrived yet and you open V6 or V16, place a beaker at the outlet of the tubing.

  - If R4 has arrived: open a valve to a CSTR.

- Turn on the pump and let water flow through the reactor.

- The flow meter should move and be responsive at this point.

- Resume normal operation.