Scientific and Technological Relevance

Pt-catalyzed NO oxidation, NO(g) + ½ O₂(g) ⇌ NO₂(g), matters both for real engines and as a clean test case for surface kinetics. In lean-burn gasoline and diesel engines (and gas turbines), the exhaust is very O₂-rich, so traditional three-way catalysts can’t reduce NOₓ effectively. Instead, aftertreatment systems like lean NOₓ traps (LNTs) and selective catalytic reduction (SCR) work much better when a significant fraction of the NO is first oxidized to NO₂. In LNTs, for example, NO₂ forms and releases nitrates on the storage material more readily than NO, so the front-end Pt oxidation catalyst basically sets the workable temperature window for the whole brick. At the same time, Pt is expensive and can deactivate under hot, oxidizing conditions, which gives a strong practical motivation to really understand how NO oxidation works on Pt surfaces and how to design catalysts that use less Pt but still hit tough NOₓ regulations.

On the science side, the Getman and Schneider paper treats this reaction as a benchmark for connecting DFT-level energetics to measured macroscopic rates. They build a coverage-dependent microkinetic model on Pt(111) and show that when the surface is mostly covered with chemisorbed oxygen (about 0.25–0.4 monolayers), the model can reproduce key experimental trends: roughly first-order behavior in NO and O₂, inhibition by NO₂, and an apparent activation energy in the right ballpark.

The work also makes it clear where simple models break down: low-coverage DFT energetics and mean-field assumptions miss important adsorbate–adsorbate interactions and give the wrong rate orders. That pushes the field toward more realistic, coverage-aware kinetic models and, eventually, tools like kinetic Monte Carlo. Because NO oxidation is relatively simple, directly tied to air-quality-driven NOₓ control, and already well-characterized on Pt in the engine and catalysis literature, it has become a very useful “test bed” both for new theoretical methods and for practical catalyst design. For instance, Epling et al. review how LNT performance depends strongly on this front-end NO → NO₂ step, underscoring the link between the mechanism studied here and real aftertreatment hardware in the field.

References for Part 7
Getman, R. B.; Schneider, W. F. “DFT-Based Coverage-Dependent Model of Pt-Catalyzed NO Oxidation,” ChemCatChem 2010, 2, 1450–1460.

Epling, W. S.; Campbell, L. E.; Yezerets, A.; Currier, N. W.; Parks, J. E. “Overview of the Fundamental Reactions and Degradation Mechanisms of Lean NOx Trap Catalysts,” Catal. Rev. 2004, 46, 163–245.