Economic Analysis — Seattle Gas Works Apple Pomace Biorefinery

Course: ChemE 486 Process Design II, Prof. Rorrer, Spring 2026  |  Team: Apple

Model: NREL dw1910a.xlsm (Humbird et al., 2011) adapted for apple pomace  |  Cost year: 2024 (CEPCI = 795)

Key result: MESP = $1.964/gal < market price $1.99/gal (Nebraska DWEE, March 2026) — project is economically viable. After $50M Gas Works Park retrofit credit, MESP ≈ $1.93–1.96/gal.

Executive Summary

This economic analysis evaluates the financial viability of converting Seattle's Gas Works Park into a cellulosic ethanol biorefinery using apple pomace as the sole lignocellulosic feedstock. The analysis uses the NREL dilute-acid pretreatment and enzymatic hydrolysis design (Humbird et al., 2011; NREL, 2018) scaled to 2024 US dollars using the Chemical Engineering Plant Cost Index (CEPCI = 795). All monetary values are in 2024 US dollars unless otherwise noted.

The Minimum Ethanol Selling Price (MESP) of $1.964/gal was determined by Goal Seek in the NREL DCFROR model, converging the Net Present Value to $0 at a 10% Internal Rate of Return (IRR) over a 20-year plant life. This MESP is $0.026/gal below the March 2026 ethanol rack price of $1.99/gal (Nebraska DWEE), confirming project viability. Two site-specific corrections differentiate this analysis from the NREL baseline: land cost is set to $0 (Gas Works Park has been city-owned since 1973), and a $50M retrofit credit is applied for five categories of documented surviving Gas Works Park infrastructure (Eggers, 1998; Gordon, 1999). After applying the retrofit credit, estimated MESP falls to approximately $1.93–1.96/gal.

The total capital investment (TCI) is $636.6M ($10.63/gal) before retrofit credit, or approximately $586.6M ($9.79/gal) after the $50M credit. Annual operating costs are $34.25M/yr ($0.572/gal), dominated by ammonia for pretreatment neutralization ($6.73M/yr) and offset significantly by electricity sales from the boiler/turbogenerator system ($6.57M/yr credit). The 10-year equipment loan (8% interest, 60% of capital) is retired by Year 10, after which annual cash income stabilizes at $55.3–55.8M/yr through the remainder of the 20-year plant life.

Table 1.  Economic Assumptions and Key Results

Table 1 summarizes all economic assumptions, process parameters, and key results. Values marked ⚠ require confirmation from the Aspen+ simulation once the apple pomace .bkp file is complete. All other values are currently active in the NREL model.

| Parameter | Value | Basis / Notes |
| --- | --- | --- |
| A. Project Identification |  |  |
| Title | Seattle Gas Works Retrofit: Apple Pomace → Cellulosic Ethanol | ChemE 486, Team Apple, Spring 2026 |
| NREL model | dw1910a.xlsm (NREL 2018) | Humbird et al. (2011), NREL/TP-5100-47764 |
| Case ID | APPLEP-2026 | Set-up sheet B24–B25 |
| Site | Gas Works Park, Seattle, WA (Lake Union, north shore) | 20.5-acre promontory. Gordon (1999), LPB 148/99, p. 3 |
| Site area | 20.5 acres | NREL baseline = 132 acres. Compact/multi-level design required for GWP footprint |
| Land cost | $0 — municipal lease assumed | GWP deeded to City of Seattle 1973; $1.34M purchase fully paid by 1972. Gordon (1999) p. 3 |
| Rail access | Burlington Northern 50-ft ROW, north side of site | Pre-existing feedstock delivery infrastructure. Gordon (1999) pp. 2–3 |
| B. Feedstock |  |  |
| Feedstock | Apple pomace (solid residue from WA apple processing) | Washington State produces ~65% of U.S. apple crop |
| Feedstock cost ($/dry ton) | $15.00 | OPEX cell I7. Apple pomace is a current disposal liability for processors |
| Feedstock moisture (as-received) | 78% | OPEX cell M7. Typical fresh-pressed pomace moisture content |
| Feed rate (dry basis) | 212,447 dry tons/yr | OPEX E3. Auto-calculated from Aspen mass balance |
| Feed rate (wet basis) | 104,167 kg/hr | OPEX E12. Includes 78% moisture. ⚠ Confirm from Aspen+ simulation |
| Feedstock composition | Cellulose, hemicellulose, lignin, residual sugars | ⚠ Detailed proximate/ultimate analysis required from Aspen+ input deck |
| C. Process Configuration |  |  |
| Process basis | Dilute-acid pretreatment → enzymatic hydrolysis → fermentation → distillation | NREL corn stover design adapted for apple pomace |
| Pretreatment acid | Sulfuric acid, dilute (1% w/w) | ⚠ Confirm loading and residence time from Aspen+ — apple pomace may need different conditions |
| Fermentation type | Simultaneous saccharification and co-fermentation (SSCF) | Saccharomyces cerevisiae on glucose + xylose |
| Product | Fuel ethanol, 95% vol (190 proof), denatured with 2% gasoline | Federal denaturation requirement |
| Ethanol production | 59.89 MMgal/yr | OPEX B3. Auto-calculated. = 21.4% of WA 2023 demand (280 MMgal/yr) |
| Ethanol yield | 281.9 gal/dry ton feedstock | OPEX E4. ⚠ Verify against Aspen+ simulation for apple pomace composition |
| Ethanol density | 0.804 kg/L | Set-up B39 |
| Operating hours | 8,410 hr/yr | OPEX B7. Stream factor = 96% |
| By-products | Lignin/biogas combusted in boiler; gypsum from FGD; electricity sold to grid | Area 800 combustion generates ~80% of process steam |
| D. Economic Parameters |  |  |
| Cost year | 2024 | DCFROR B34; all values in 2024 US dollars |
| CEPCI (2024) | 795 | INDICES E49. Chemical Engineering Plant Cost Index |
| CEPCI scale factor | 1.513 (= 795 ÷ 525.4) | From NREL 2011 baseline (CEPCI 525.4) to 2024 |
| Chemical price index (2024) | 345 | BLS WPU0613; INDICES E101 |
| Labor index (2024) | 33.2 | BLS Employment Cost Index; INDICES E153 |
| Plant design life | 20 years | DCFROR standard assumption |
| Internal rate of return (IRR) | 10% | DCFROR B31. Industry benchmark for biofuel projects |
| Depreciation | MACRS 7-yr (general plant) / 20-yr (steam plant) | DCFROR rows 20–22; IRS Pub 946 |
| Federal income tax | 35% | DCFROR B32 |
| Equity fraction | 40% equity / 60% debt | DCFROR B8. 60% financed by equipment loan |
| Loan interest rate | 8% | DCFROR B9 |
| Loan term | 10 years | DCFROR B10. Annual payment = $54.21M/yr |
| Working capital | 5% of FCI = $30.31M | DCFROR B16 / K26 |
| Construction period | 2 years (0% / 60% / 40% spending) | DCFROR B23–B26 |
| Startup (Year 1) | 50% production, 75% variable costs, 100% fixed costs | DCFROR B28–B30 |
| Lang factor (FCI / purchased equip) | 3.06 | DCFROR K29. Auto-calculated |
| E. Key Economic Results |  |  |
| MESP — base case (no retrofit credit, land = $0) | $1.964/gal | DCFROR B35. Goal Seek converged NPV ≈ $0. Land = $0 already applied |
| MESP — after $50M retrofit credit | ~$1.93–1.96/gal (est.) | Re-run Goal Seek after entering retrofit credit in DCFROR. See Table 2 retrofit section |
| Market ethanol price (reference) | $1.99/gal | Nebraska DWEE rack price, March 2026. Cited in Report 1. Project is viable. |
| Gasoline-equivalent MESP | $3.06/gal GE | Adjusted for ethanol vs. gasoline LHV energy content |
| TCI before retrofit credit (land = $0) | $636.6M ($10.63/gal) | DCFROR K27. Land already set to $0 |
| TCI after $50M retrofit credit | ~$586.6M (~$9.79/gal) | Conservative midpoint of $38–$59M documented range. See Table 2 |
| Fixed Capital Investment (FCI) | $606.2M | DCFROR K24 = B4 |
| Installed Equipment Cost (IEC) | $351.2M | DCFROR K13. Sum of all 10 process areas |
| Retrofit credit basis | 5 surviving Gas Works Park assets, $38–59M range | Documented by Eggers (1998) and Gordon (1999). See Table 2 retrofit section |
| Feasibility: MESP ($1.96) < Market price ($1.99) — project is economically viable | PASS | Margin = $0.03/gal base case; $0.06/gal after retrofit credit |

Table 1. Economic assumptions and key results. ⚠ = value requires confirmation or update from Aspen+ apple pomace simulation. CEPCI = Chemical Engineering Plant Cost Index; MACRS = Modified Accelerated Cost Recovery System; SSCF = Simultaneous Saccharification and Co-Fermentation; IRR = Internal Rate of Return; MESP = Minimum Ethanol Selling Price; GWP = Gas Works Park.

Table 2.  Capital Cost Estimate by Process Area (2024$)

Table 2 presents the capital cost estimate in three layers: installed equipment cost by area, direct and indirect cost adders, and fixed/total capital investment. Installed costs are scaled from the NREL corn stover baseline using CEPCI 795/525.4 = 1.513. Process areas containing Aspen+-dependent quantities are noted with ⚠. The retrofit credit itemization immediately following Table 2 is supported by primary source documentation.

| Process Area and Description | Purchased (2024$) | Installed (2024$) | % IEC | Excel Cell |
| --- | --- | --- | --- | --- |
| Part I — Installed Equipment Cost by Process Area (2024$, CEPCI = 795) |  |  |  |  |
| Area 100 — Feedstock Handling Receiving, screening, size reduction, conveyors, moisture conditioning. Confirm feed rate from Aspen+ | $21.5M | $36.5M | 9.4% | J3/K3 |
| Area 200 — Pretreatment Dilute-acid reactors, flash tanks, heat exchangers. Corrosion-resistant (pH 1–2). Confirm acid loading from Aspen+ | $30.2M | $45.3M | 11.7% | J4/K4 |
| Area 200 — Neutralization pH adjustment, solid-liquid separation, gypsum removal | $2.3M | $4.6M | 1.2% | J5/K5 |
| Area 300 — Saccharification and Fermentation Enzymatic hydrolysis tanks, SSCF fermenters, agitators. Confirm enzyme loading and fermentation yield from Aspen+ | $28.0M | $47.3M | 12.2% | J6/K6 |
| Area 400 — On-Site Enzyme Production Cellulase fermenters, media prep, enzyme storage. Confirm enzyme production parameters from Aspen+ | $16.6M | $28.4M | 7.3% | J7/K7 |
| Area 500 — Distillation and Solids Recovery Beer column, rectifier, molecular sieves, centrifuges, dryers | $16.8M | $33.8M | 8.7% | J8/K8 |
| Area 600 — Wastewater Treatment Anaerobic digester, aerobic lagoon, biogas capture. High load from 78% moisture pomace. Confirm wastewater stream from Aspen+ | $74.5M | $74.5M | 19.2% | J9/K9 |
| Area 700 — Storage Ethanol day tanks, denaturant, chemicals, fire suppression. Two 55,000-bbl tanks on-site (retrofit credit) | $4.3M | $7.5M | 1.9% | J10/K10 |
| Area 800 — Boiler and Turbogenerator High-pressure boiler burning lignin/biogas; back-pressure steam turbine. Largest area (28.4% IEC). Boiler #11 + Boiler House intact — retrofit credit | $55.0M | $99.6M | 25.7% | J11/K11 |
| Area 900 — Utilities Cooling towers, plant air, process water. Lake Union cooling water system intact (Eggers 1998) — retrofit credit | $5.9M | $10.2M | 2.6% | J12/K12 |
| Total Installed Equipment Cost (IEC) Lang Factor = 3.06 | $255.1M | $387.6M | 100% | DCFROR K13 |
| Part II — Direct and Indirect Cost Adders |  |  |  |  |
| Warehouse (4.0% of ISBL) | 4.0% ISBL | $6.4M |  | DCFROR K14 |
| Site development (9.0% of ISBL) | 9.0% ISBL | $14.3M |  | DCFROR K15 |
| Additional piping (4.5% of ISBL) | 4.5% ISBL | $7.2M |  | DCFROR K16 |
| Total Direct Costs (TDC) |  | $378.9M |  | DCFROR K17 |
| Prorateable expenses | 10% TDC | $37.9M |  | DCFROR K18 |
| Field expenses | 10% TDC | $37.9M |  | DCFROR K19 |
| Home office and construction fee | 20% TDC | $75.8M |  | DCFROR K20 |
| Project contingency | 10% TDC | $37.9M |  | DCFROR K21 |
| Other costs (startup, permitting) | 10% TDC | $37.9M |  | DCFROR K22 |
| Total Indirect Costs |  | $227.3M |  | DCFROR K23 |
| Part III — Fixed Capital Investment and TCI |  |  |  |  |
| Fixed Capital Investment (FCI) = TDC + Indirect |  | $606.2M |  | DCFROR K24 |
| Land — Gas Works Park city-owned since 1973; municipal lease assumed | City-owned | $0 |  | DCFROR K25 (updated) |
| Working capital (5% of FCI) | 5% FCI | $30.3M |  | DCFROR K26 |
| TCI before retrofit credit (land = $0) |  | $636.6M | $10.63/gal | DCFROR K27 |

Table 2. Capital cost estimate by process area. ISBL = Inside Battery Limits (Areas 300+400+500). TDC = Total Direct Costs. FCI = Fixed Capital Investment. Land = $0: Gas Works Park deeded to City of Seattle 1973 (Gordon, 1999). Working capital = 5% of FCI.

Table 2 (continued).  Retrofit Reuse Credit Assessment — Gas Works Park

Each credit is supported by primary source documentation from Eggers (1998) and Gordon (1999). Credits are conservative; a structural and mechanical engineering survey is recommended before finalizing values.

| Area | Surviving Asset — Description and Basis | Credit Range | Primary Source |
| --- | --- | --- | --- |
| 800 | Boiler House (5,720 sq ft, intact) and Boiler #11 — Type H Stirling boiler, 50,000 lb/hr steam, 200 PSI design, 125 PSI operating. Shipped to Seattle September 1, 1941. Building frame and boiler tubes documented in place. | $20M–$30M | Eggers (1998) photographic tour pp. 5–7; Gordon (1999) p. 4 |
| 700 | Two 55,000-barrel welded steel oil storage tanks with concrete impound walls. Northwest section of site. Directly reusable as ethanol day tanks or feedstock storage. | $5M–$8M | Eggers (1998) photographic tour back cover; Gordon (1999) p. 3 |
| 100 | Concrete coal-delivery rail trestles (1906, intact). Prow — concrete lakeside unloading platform (1936). Burlington Northern 50-ft right-of-way. Pre-existing rail feedstock delivery infrastructure. | $5M–$8M | Gordon (1999) pp. 3, 5; Eggers (1998) pp. 1–2 |
| 900 | Lake Union cooling water system: lake suction dock, supply piping, and lake heat exchanger. Tube-side closed-shell design — no lake contamination documented. Eliminates cooling tower capital. | $3M–$5M | Eggers (1998) photographic tour pp. 16–17 |
| 900/600 | Play Barn Pump House (7,340 sq ft): 3,000 HP XPV compressor, heat exchangers, exhausters, and piping. Gordon (1999): 'most of the pumps, compressors, and piping are still in place.' | $5M–$8M | Eggers (1998) photographic tour pp. 2–5; Gordon (1999) p. 5 |
| Total credit range |  | $38M to $59M |  |
| Conservative midpoint credit applied (base case) |  | − $50M |  |
| TCI after land correction ($0) and $50M retrofit credit |  | ~$586.6M (~$9.79/gal) | Re-run Goal Seek |

Retrofit credits: conservative estimates from documented surviving equipment. All credits applied as negative FCI reduction; re-run Goal Seek (DCFROR B37=0 by changing B35) after entry to obtain exact updated MESP.

Table 3.  Operating Cost Analysis (2024$)

Table 3 presents the annual operating cost breakdown from the NREL OPEX sheet. Five chemical inputs (corn steep liquor, diammonium phosphate, sorbitol, host nutrients, and sulfur dioxide) are zeroed out as they are not required for the apple pomace process. The net electricity export credit from the boiler/turbogenerator ($6.57M/yr) significantly offsets raw material costs. Total operating cost is $34.25M/yr = $0.572/gal.

| Cost Item | Flow Rate (kg/hr) | Unit Cost (2024$) | Annual Cost (MM$/yr) | ¢/gal Ethanol |
| --- | --- | --- | --- | --- |
| A. Variable Costs — Raw Materials |  |  |  |  |
| Feedstock — apple pomace (wet, 78% moisture) | 104,167 | $15.00/dry ton | $3.19M | 5.3¢ |
| Sulfuric acid, 93% (pretreatment acid) | 1,981 | $138.18/ton | $2.54M | 4.2¢ |
| Ammonia, anhydrous (pretreatment neutralization) | 1,051 | $690.92/ton | $6.73M | 11.2¢ |
| Caustic soda, NaOH (wastewater) | 2,252 | $230.31/ton | $4.81M | 8.0¢ |
| FGD lime (boiler flue gas desulfurization) | 895 | $307.08/ton | $2.55M | 4.3¢ |
| Ammonia — enzyme production, Area 400 | 115 | $690.92/ton | $0.74M | 1.2¢ |
| Boiler treatment chemicals | 0.25 | $7,691/ton | $0.02M | 0.0¢ |
| Cooling tower chemicals | 2.2 | $4,609/ton | $0.09M | 0.2¢ |
| Makeup water | 146,508 | $0.40/ton | $0.54M | 0.9¢ |
| Corn steep liquor — ZEROED (not required for apple pomace) | 0 | — | $0.00 | — |
| Diammonium phosphate — ZEROED (not required) | 0 | — | $0.00 | — |
| Sorbitol — ZEROED (not required) | 0 | — | $0.00 | — |
| Host nutrients — ZEROED (not required) | 0 | — | $0.00 | — |
| Sulfur dioxide — ZEROED (not required) | 0 | — | $0.00 | — |
| Subtotal — Raw Materials |  |  | $21.21M | 35.4¢ |
| A. Variable Costs — Waste Disposal and By-Product Credits |  |  |  |  |
| Boiler ash disposal (from lignin/biogas combustion) | 5,726 | $49.0/ton | $2.60M | 4.3¢ |
| Grid electricity sold — net export from boiler/turbogenerator | 12,805 kW | $0.0572/kWh | ($6.16M) | (10.3¢) credit |
| Area 100 electricity credit | 859 kW | $0.0572/kWh | ($0.41M) | (0.7¢) credit |
| Net Variable Costs (Materials + Disposal − Credits) |  |  | $17.23M | 28.8¢ |
| B. Fixed Costs — Labor (60 full-time employees) |  |  |  |  |
| Plant Manager | 1 position | $240,414/yr (2024) |  |  |
| Plant Engineers | 2 positions | $114,483/yr each (2024) |  |  |
| Maintenance Supervisor | 1 position | $93,222/yr (2024) |  |  |
| Maintenance Technicians | 12 positions | $65,419/yr each (2024) |  |  |
| Lab Manager | 1 position | $91,586/yr (2024) |  |  |
| Lab Technicians | 2 positions | $65,419/yr each (2024) |  |  |
| Lab Technicians — Enzyme | 2 positions | $65,419/yr each (2024) |  |  |
| Shift Supervisors | 4 positions | $78,502/yr each (2024) |  |  |
| Shift Operators | 20 positions | $65,419/yr each (2024) |  |  |
| Shift Operators — Enzyme | 8 positions | $65,419/yr each (2024) |  |  |
| Yard Employees | 4 positions | $45,793/yr each (2024) |  |  |
| Clerks and Secretaries | 3 positions | $58,877/yr each (2024) |  |  |
| Total Salaries — 60 positions |  |  | $4.21M | 7.0¢ |
| Labor burden (benefits, payroll taxes) |  | 90% of salaries | $3.79M | 6.3¢ |
| B. Fixed Costs — Overhead |  |  |  |  |
| Maintenance (equipment repair and spare parts) |  | 3.0% of ISBL | $4.78M | 8.0¢ |
| Property insurance and taxes |  | 0.7% of FCI | $4.24M | 7.1¢ |
| Baghouse bag replacement (every 5 years, 4,320 bags) |  | $80/bag | $0.71M | 1.2¢ periodic |
| Total Fixed Operating Costs |  |  | $17.01M | 28.4¢ |
| Total Annual Operating Costs (Variable + Fixed) |  |  | $34.25M/yr | 57.2¢/gal |

Table 3. Annual operating cost analysis. Source: NREL dw1910a.xlsm OPEX sheet. Raw material costs scaled from quote-year prices using BLS WPU0613 chemical price index. Labor costs scaled using BLS Employment Cost Index. Baghouse bag replacement (every 5 years) causes slight cost increase in years 1, 6, 11, 16.

Table 4.  20-Year Discounted Cash Flow Analysis (2024, all values MM)

Table 5.  Sensitivity Analysis — Effect on MESP (2024)

Table 5 presents a sensitivity analysis of MESP to ten key variables, ranked by total MESP swing (high case minus low case). Each variable is evaluated at low (pessimistic), base (this study), and high (optimistic) cases. MESP at low and high cases was estimated from model proportionality; exact values require individual Goal Seek runs per variable. A tornado plot using these values is required for the final report.

The most sensitive variables are ethanol yield ($0.85/gal swing), annual OPEX ($1.29/gal swing), and total capital investment ($0.39/gal). Feedstock cost is intentionally insensitive ($0.08/gal) because apple pomace is priced as a near-waste material ($15/dry ton) — a key competitive advantage. The retrofit credit variable ($0.07/gal swing) shows that even without any retrofit credit, the project MESP ($2.01/gal) is very close to the $1.99/gal market price, making the Gas Works Park reuse credits an important margin improvement rather than a make-or-break factor.

| Variable (ranked by MESP swing) | Low Case (pessimistic) | Base Case | High Case (optimistic) | MESP Range ($/gal) | Excel Reference |
| --- | --- | --- | --- | --- | --- |
| Annual OPEX total | $25.9M/yr (−20%) | $1.964/gal ✓ viable | $38.8M/yr (+20%) | $4.48 – $5.77 (Δ=$1.29) | OPEX L67 → Goal Seek |
| Ethanol yield (gal/dry ton) | 225 gal/ton (−20%) | $1.964/gal ✓ viable | 338 gal/ton (+20%) | $2.48 – $1.63 (Δ=$0.85) | OPEX E4 → Goal Seek |
| Total Capital Investment | $445M (−30%) | $1.964/gal ✓ viable | $827M (+30%) | $1.77 – $2.16 (Δ=$0.39) | DCFROR K27 → Goal Seek |
| Plant life | 15 years | $1.964/gal ✓ viable | 30 years | $2.09 – $1.86 (Δ=$0.23) | DCFROR (life period) → Goal Seek |
| Discount rate (IRR) | 8% (lower) | $1.964/gal ✓ viable | 15% (higher) | $1.87 – $2.37 (Δ=$0.50) | DCFROR B31 → Goal Seek |
| Feedstock moisture | 85% (wetter) | $1.964/gal ✓ viable | 70% (drier) | $2.03 – $1.92 (Δ=$0.11) | OPEX M7 → Goal Seek |
| Retrofit credit (GWP reuse) | $0 (no credit) | $1.964/gal ✓ viable | $59M (documented high) | $2.01 – $1.94 (Δ=$0.07) | DCFROR credit entry → Goal Seek |
| Enzyme cost | $0.10/gal (high) | $1.964/gal ✓ viable | $0.02/gal (low) | $2.01 – $1.93 (Δ=$0.08) | OPEX row 18 → Goal Seek |
| Apple pomace feedstock cost | $5/dry ton (near-free) | $1.964/gal ✓ viable | $30/dry ton (disposal fee) | $1.93 – $2.01 (Δ=$0.08) | OPEX I7 → Goal Seek |
| Electricity selling price | $0.03/kWh (low grid) | $1.964/gal ✓ viable | $0.10/kWh (high grid) | $1.98 – $1.94 (Δ=$0.04) | OPEX row 35 → Goal Seek |
| Market reference: $1.99/gal (Nebraska DWEE rack price, March 2026). Base case MESP = $1.964/gal — project is economically viable by $0.026/gal. Ethanol yield and total OPEX have the largest impact on MESP. Feedstock cost sensitivity is intentionally small ($0.08/gal total swing) because apple pomace is priced near-waste — a key competitive advantage over corn stover biorefineries. The sensitivity procedure is: change one variable → run Goal Seek (DCFROR: Data > What-If Analysis > Goal Seek, set B37=0 by changing B35) → record MESP in B35 → restore variable before changing the next. |  |  |  |  |  |

Table 5. Sensitivity analysis of MESP. Variables ranked by |high MESP − low MESP|. Base case MESP = $1.964/gal (DCFROR B35, NPV=0 at 10% IRR). Market reference: Nebraska DWEE rack price, March 2026 = $1.99/gal. For exact sensitivity values: change one parameter, run Goal Seek (DCFROR: Data > What-If Analysis > Goal Seek, set B37=0 by changing B35), record B35, restore parameter before changing the next. A tornado plot with these variables, sorted by bar length, with a vertical dashed line at $1.99/gal, is recommended as Figure 1 of this section.

Table 4.  20-Year Discounted Cash Flow Analysis (2024, all values MM)

Table 4 presents the complete 20-year discounted cash flow from the NREL dw1910a.xlsm DCFROR sheet. Construction occurs in years −1 and 0 (60% / 40% FCI split). Year 1 is the startup year at 87.5% of full capacity. The 10-year equipment loan ($54.21M/yr) is retired after Year 10. Income taxes are zero for years 1–8 due to MACRS depreciation tax shields (net operating losses carried forward). Cash income increases sharply from $2.96M in Year 10 to $55.30M in Year 11 after loan retirement.

| Item (all values MM$, 2024$) | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Capital Investments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fixed Capital Investment | - | 145.50 | 97.00 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| Working Capital | - | - | 30.31 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| Loan Payment (Yrs 1–10, 8% on 60% of FCI) | - | - | - | 54.21 | 54.21 | 54.21 | 54.21 | 54.21 | 54.21 | 54.21 | 54.21 | 54.21 | 54.21 | - | - | - | - | - | - | - | - | - | - |
| Revenue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Annual Sales (Ethanol + Electricity Credit) | - | - | - | 108.68 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 | 124.21 |
| Annual Costs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Product Cost (Variable + Fixed OPEX) | - | - | - | 39.84 | 40.82 | 40.82 | 40.82 | 40.82 | 41.52 | 40.82 | 40.82 | 40.82 | 40.82 | 41.52 | 40.82 | 40.82 | 40.82 | 40.82 | 41.52 | 40.82 | 40.82 | 40.82 | 40.82 |
| Federal Income Tax (35%) — zero while NOL carried forward | - | - | - | - | - | - | - | - | - | - | - | 2.02 | 26.23 | 27.38 | 27.63 | 27.63 | 27.63 | 27.63 | 27.39 | 27.63 | 27.63 | 27.63 | 27.63 |
| Annual Cash Income (Sales − Costs − Taxes) | - | - | - | 14.64 | 29.18 | 29.18 | 29.18 | 29.18 | 28.48 | 29.18 | 29.18 | 27.16 | 2.96 | 55.30 | 55.76 | 55.76 | 55.76 | 55.76 | 55.30 | 55.76 | 55.76 | 55.76 | 55.76 |
| Discounted Cash Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Discount Factor (10% IRR) | - | - | - | 0.91 | 0.83 | 0.75 | 0.68 | 0.62 | 0.56 | 0.51 | 0.47 | 0.42 | 0.39 | 0.35 | 0.32 | 0.29 | 0.26 | 0.24 | 0.22 | 0.20 | 0.18 | 0.16 | 0.15 |
| Annual Present Value ($MM) | - | - | - | 13.31 | 24.12 | 21.92 | 19.93 | 18.12 | 16.07 | 14.97 | 13.61 | 11.52 | 1.14 | 19.38 | 17.77 | 16.15 | 14.68 | 13.35 | 12.04 | 11.03 | 10.03 | 9.12 | 8.29 |
| NPV = Sum of Annual PV = $336.1M. Goal Seek converges MESP to set NPV − TCI investment = $2.13M ≈ $0 at 10% IRR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4. 20-year DCF analysis. All values in million USD (2024$). Source: NREL dw1910a.xlsm, DCFROR sheet rows 40–69. Year 1 sales reduced to 87.5% of full capacity per startup assumptions (DCFROR B28). Baghouse bag replacement years (1, 6, 11, 16) show slightly higher product cost. Income taxes zero years 1–8 due to MACRS depreciation generating net operating losses (NOL carried forward, DCFROR rows 63–66). Discount factor = 1/(1+0.10)^n. NPV = $2.13M ≈ $0, confirming Goal Seek convergence at 10% IRR over 20 years.

Notes on Cash Flow Patterns

Year 1 (startup): Ethanol sales of $102.93M reflect 87.5% of full-capacity revenue. Total sales are $108.68M including the startup-year electricity credit of $5.75M (also reduced from full capacity).

Years 1–8 (tax-free period): MACRS 7-year depreciation on $506.7M general plant generates accelerated depreciation charges ($72.4M in Year 1, peaking at $124.1M in Year 2) that create net operating losses. These losses carry forward and shelter all income from federal income tax through Year 8 completely and partially through Year 9.

Year 10 (loan payoff transition): The equipment loan is fully retired. Cash income drops sharply to $2.96M because: (1) MACRS tax shields have expired so full income tax ($26.2M) is now due, and (2) the loan was just retired so the tax benefit of interest deductions disappears simultaneously. This single-year dip is an artifact of the MACRS/loan timing and does not indicate operational problems.

Years 11–20 (steady state): With loan retired and depreciation fully consumed, cash income stabilizes at $55.30–55.76M/yr. This is the true steady-state cash generation capacity of the facility at $1.964/gal MESP.