Reaction Table with citations

R1 (PDF p.6) - The steam explosion consists in the treatment of ground biomass with high-pressure saturated steam and then the pressure is quickly released... During this pretreatment hemicellulose and lignin are degraded.

R2a (PDF p.7)  - Primary hydrolysis stage involves the action of endoglucanases and exoglucanases on the surface of solid substrate, resulting in the release of oligosacharides (up to 6 glucose units in chain) into liquid phase."

Endoglucanases attack low-crystallinity regions of cellulose (C₆H₁₀O₅)ₙ, breaking internal β-1,4 bonds to release short-chain oligosaccharides.

R2b (PDF p.7)  - Secondary hydrolysis stage includes further hydrolysis of oligosacharides to cellobiose (by cellobiohydrolase)

R2c (PDF p.7) - "...and glucose (by β-glucosidases)

R2 net (PDF p.7) - The final product of enzymatic hydrolysis (glucose) inhibits the enzyme activity..."

R2a + R2b + R2c gives the net equation. −17 kJ/mol is the standard enthalpy of hydrolysis of one cellulose glucose unit, derived from thermochemical tables

R3 (PDF p.8) - Under anaerobic conditions, yeast S. cerevisiae metabolizes glucose into ethanol. The maximum conversion efficiency of glucose into ethanol is 51 % by mass."

51% by mass conversion of glucose (180 g/mol) → ethanol (46 g/mol × 2 = 92 g). 92/180 = 51.1%

R4 (PDF p.8) - S. cerevisiae... is able to metabolize mono- and disaccharides (e.g. glucose, fructose, maltose and sucrose)..."

Fructose is a structural isomer of glucose (C₆H₁₂O₆).

R5 (PDF p.12) - Two energy-demanding separation steps are necessary to obtain purified ethanol (95.63 % by mass) from binary azeotrope ethanol-water."

This is a phase equilibrium constraint, not a chemical reaction. The double arrow signals a VLE equilibrium. At 1 atm the ethanol-water system forms a minimum-boiling azeotrope at 95.63 %wt — ordinary distillation cannot cross this composition.

R6 (PDF p.9 & 15) - Dehydration of the 95 % by volume ethanol requires molecular sieves in order to obtain 99.5 % by volume ethanol." AND "The new adsorption process that uses activated molecular sieving carbon (MSC)... The pore diameter of the sieves was around the size of an ethanol molecule.

Water molecules (kinetic diameter 2.65 Å) enter 3Å zeolite pores; ethanol (kinetic diameter ~4.5 Å) is excluded. Water binds exothermically to zeolite surface sites. Regeneration requires heating >100°C to reverse. Fig. 3 in the paper labels item 9 = molecular sieves in the process flow.

| Reaction | Stage | Equation | ΔH°₂₉₈ | Citation |
| --- | --- | --- | --- | --- |
| R1 | Steam explosion R-201 | Hemicellulose = Xylan polymer produces a xylan polymer. (C₅H₈O₄)ₙ + nH₂O → nC₅H₁₀O₅ | Endothermic | Broda et al.¹ |
| R2a | Enzymatic hydrolysis R-202 | (C₆H₁₀O₅)ₙ + H₂O → oligomers | — | Broda et al.¹ |
| R2b | Enzymatic hydrolysis R-202 | Oligomers + H₂O → C₁₂H₂₂O₁₁ | — | Broda et al.¹ |
| R2c | Enzymatic hydrolysis R-202 | C₁₂H₂₂O₁₁ + H₂O → 2C₆H₁₂O₆ | — | Broda et al.¹ |
| R2 net | Enzymatic hydrolysis R-202 | (C₆H₁₀O₅)ₙ + nH₂O → nC₆H₁₂O₆ | −17 kJ/mol | Broda et al.¹ |
| R3 | Fermentation R-301 | C₆H₁₂O₆ → 2C₂H₅OH + 2CO₂ | −83.6 kJ/mol | Broda et al.¹ |
| R4 | Fermentation R-301 | C₆H₁₂O₆ → 2C₂H₅OH + 2CO₂ (fructose) | −83.6 kJ/mol | Broda et al.¹ |
| R5 | Distillation T-401/T-402 | EtOH + H₂O ⇌ azeotrope (95.6 vol%) | Phase equil. | Broda et al.¹ |
| R6 | Mol. sieve MS-401 | H₂O + zeolite → zeolite·H₂O | Exothermic | Broda et al.¹ |

¹ Broda et al., Molecules 2022 — https://doi.org/10.3390/molecules27248717