Mark A. Heien

Volumetric Analysis Part 1 was on 1/24/2023 and Part 2 was carried out on 1/31/2023

- Purpose: Objective:

  - The purpose of the experiment is to standardize an NaOH solution with the help of a prepared acid solution utilizing titration. In the process of titration, a basic solution is gradually added to the acidic solution until complete neutralization is obtained. The ‘end point’ of the titration is detected with the help of an indicator (phenolphthalein) as color of the solution changes upon neutralization.  By measuring the volume of the titrant required to reach the ‘end point’, it is possible to relate the concentration of the acid to the concentration of the base. In this manner, the unknown concentration can be expressed through the known concentration. The concentration determination is repeated several times to improve the precision of the measurements and to estimate the experimental error.

Procedure:

Procedure Part 1 – 10/19/2022

- Utilizing a 1 L plastic bottle and cap and a 250 mL plastic bottle and cap, the 1 L plastic bottle was filled approximately one-quarter full of DI water. 10 ML of a 0.12M NaOH was added to the 1 L plastic bottle. Next more DI water was added for a total volume of ~500 mL. The bottle was capped and the solution vigorously to get a completely uniform mixture.

½ of the solution was placed in a 250 mL plastic bottle and a cap was placed on the bottle. The bottle was labeled and stored for the following week’s experiment. A clean burette was filled with the NaOH solution with an initial volume was recorded.

Weighting paper was utilized to weigh between 0.4 and 0.5 grams of KHC8H4O4 (abbreviated as KHP) using an analytical balance, and the mass was recorded. The KHP was transferred to the Erlenmeyer flasks.  The KHP was dissolved with approximately 50 ML of DI water and 3 drops of phenolphthalein indicator were added to the flask.

The flask was then placed under the buret and the acid solution was titrated with NaOH solution in the burette until the solution turned pink. The end reading in the buret was recorded.
The titration was repeated 2 times enduring the NaOH solution volume was constant within 1.5%

Molar concentration was calculated and recorded.

Procedure Part 2 – 10/26/2022

- A beaker of unknown acid was obtained from the instructor. A similar process was utilized for the 2nd part of the trial. Utilizing the NaOH solution made on the first day of lab. First the dilute NaOH solution from your 250 mL bottle to a large 500-mL Erlenmeyer and swirl the solution to ensure a uniform mixture. Wash, condition, and fill the buret with your dilute NaOH solution.

A 10-mL pipet was used to deliver (10.0 mL) of the unknown acid and was placed in a 125 mL flask, mixed with 40 mL of DI water to increase the volume of solution, and make the color changes easier to detect. 3 drops of phenolphthalein indicator were added to the solution. A second and 3rd flask were prepared in the same manner. The Erlenmeyer flask with the NaOH solution was swirled to ensure a homogeneous mixture, then the buret was used to titrate the flasks containing the unknown acid solutions until the solution turned slightly pink.

The amounts in the buret were recorded to calculate the percent difference for trials 1-3 for the hydrochloric acid. If the percent difference is greater than 1.0%, carry out additional trials until at least two trials agree to within 1.0%. The differences were recorded. Proper lab procedures were utilized to clean all the lab equipment, counter, as well as proper hygiene.

Data:

Data Table for the Standardization of NaOH Solution

|  | Trial 1 | Trial 2 | Trial 3* |
| --- | --- | --- | --- |
| Mass of KHP used | 0.4870 g | 0.4303 g | N/A g |
| Initial buret reading | 0.35 mL | 0.39 mL | N/A mL |
| Final buret reading | 19.40 mL | 17.21 mL | N/A mL |
| Volume of NaOH used | 19.05 mL = 0.01905 L | 17.21 mL = 0.01721 L | N/A |

Results Table for the Standardized NaOH Solution

|  | Trial 1 | Trial 2 | Trial 3* |
| --- | --- | --- | --- |
| Molarity of NaOH | 0.1252M | 0.1253M | N/A |
| Average Molarity of NaOH | 0.1253 |  |  |
| Percent Difference | 0.08% |  |  |

0.1253 M NaOH will be utilized as the titration standard.

Data Table for the Hydrochloric Acid with an Unknown Molar Concentration

|  | Trial 1 | Trial 2 | Trial 3* |
| --- | --- | --- | --- |
| Volume of acid used | 10.0 mL | 10.0 mL | N/A |
| Initial buret reading | 0.10 mL | 17.40 mL | N/A |
| Final buret reading | 17.40 mL | 34.95 mL | N/A |
| Volume of NaOH used | 19.30 mL = 0.01930 L | 17.55 mL = 0.01755 L | N/A |

Results Table for the Hydrochloric Acid with an Unknown Molar Concentration     Unknown Number: Win23 #009

|  | Trial 1 | Trial 2 | Trial 3* |
| --- | --- | --- | --- |
| Molarity of HCl(aq) | 0.0724 | 0.0714 | N/A |
| Average Molarity of HCl(aq) | 0.0719 M |  |  |
| Percent Difference | 1.39% |  |  |

Phenolphthalein solution was utilized as an acid-base indicator to monitor the progress of the reactions.

Calculations

Part 1 experiment 1/24/2023

NaOH(aq) + KHC8H4O4(aq) = KNaC8H4O4(aq) + H2O(l)					KHP abbreviated for KHC8H4O4(aq)

NaOH reacts with KHP in a 1:1 molar ratio.

Trial #1

- 0.4870 g KHP X 1 mol KHP204.22 g KHP× 1 mol NaOH1 mol KHP=0.002385 ÷0.1905 =0.1252 moles NaOH

Trial #2

- 0.4303 KHP x 1 mol KHP204.22 g KHP ×1 mol NaOH1 mol KHP=0.002107 ÷0.01682 =0.1253 moles NaOH

Average Molarity of NaOH from trials 1 & 2 will be utilized for part 2: 0.1253M NaOH

Part 2 experiment calculations 1/31/2023

  - NaOH reacts with HCl in a 1:1 molar ratio

  - Average Molarity of NaOH from trials 1 & 2 will be utilized for part 2: 0.1253M NaOH

  - The balance condition at the end point of the titration is now used to determine the concentration of the unknown,

Trial #1

0.010 L HCl × 0.1253M = 0.001253M ÷0.01730L= 0.0724 M HCl

Trial #2

0.010 L HCL ×0.1253M=0.001253M ÷ 0.01755L=0.0714 M HCl

Results

When the color of the solution begins to change color and the new color persists for at least 30 seconds, this means the solution reaches the endpoint. For phenolphthalein, the color will change to light pink color. In the first trial NaOH, a strong base with a ph of approximately 13 was neutralized with KHP to approximately a ph of 8.  This information was then utilized to calculate the average molarity of the NaOH solution which came to 0.1253M. In the second trial the NaOH ph13 was used to neutralize the acid (HCl – ph 1.75 and calculate the molarity of the HCl which averaged out to be 0.0179M

Discussion

The purpose of the experiment was to standardize a sodium hydroxide solution to be used in titration in the presence of an indicator, phenolphthalein, to determine the concentration of an unknown sample consisting of potassium hydrogen phthalate and neutral diluents. Due to the simple nature of the experiment, it was crucial to have accurate measurements and clean equipment.

Conclusion

After 2 titration trials it was determined that the average molarity of the NaOH was between 0.1252 M and 0.1253 M. The average molarity was used 0.1253 M to proceed a in the second part of the experiment as it had only a 0.08% difference in the molarity between trial #1 and trial #2. This was then used to neutralize an unknown molarity of HCl and the experimental data showed a concentration of 0.0719 M HCl.